NetLinx Module Interface Specification

for

TOA D-901 Digital Mixer

TABLE OF CONTENTS

Introduction	3

Overview	3

Implementation	4

Command Interface	5

String Feedback	11

Device Notes	14

Programming Notes	14

Adding Functions to Modules	15

Commands to the device	15

Additional Feedback from the device	15

LIST OF TABLES

Table 1 – Send Command Definitions	10

Table 2 – String Feedback Definitions	13

| ###### Date | Initials | Comments |
| --- | --- | --- |
| 09-09-2004 | CN | v1.0 Initial release |

## Introduction

This is a reference manual to describe the interface provided between an AMX NetLinx system and a TOA D-901. The D-901 supports an RS-232 serial protocol. The interface was tested using version 2.0 of the firmware. The required communication settings are a baud rate of 9600, 8 data bits, 1 stop bit, no parity, and no handshaking. The cable for this device is FG#10-752.

## Overview

The COMM module translates between the standard interface described below and the D-901 serial protocol. It parses the buffer for responses from the device, sends strings to control the device, and receives commands from the UI module or telnet sessions.

A User Interface (UI) module is also provided. This module uses the standard interface described below and parses the string responses for feedback.

The following diagram gives a graphical view of the interface between the interface code and the NetLinx module.

TOA

D-901

NetLinx

COMM Module

NetLinx

UI Module

Virtual

Device

## Implementation

To interface to the AMX TOA_D901_COMM module, the programmer must perform the following steps:

- Define the device ID for the D-901 that will be controlled.

- Define the virtual device ID that the TOA_D901_COMM COMM module will use to communicate with the main program and User Interface. NetLinx virtual devices start with device number 33001.

- If a touch panel interface is desired, a touch panel file TOA_D901.TP4 and TOA_D901_UI.axs have been created for testing.

- The NetLinx TOA_D901_COMM module must be included in the program with a DEFINE_MODULE command. This command starts execution of the module and passes in the following key information: the device ID of the D-901 to be controlled, and the virtual device ID for communicating to the main program.

An example of how to do this is shown below.

DEFINE_DEVICE

dvDevice		= 5001:1:0	// The D-901 connected to the NetLinx on 1st RS-232 port

dvTouchPanel 	= 128:1:0	// The touch panel used for output

vdvDevice       	= 33001:1:0	// The virtual device use for communication between the

// Comm module interface and User_Interface (UI) module interface

DEFINE_VARIABLE

//Define arrays of button channels used on your own touch panel

integer nTP_BUTTONS[]={1,2,3,4,5,6,7}

integer nTP_TEXT[]={1,2,3}

integer nTP_LEVELS[]={1,2}

DEFINE_START	// Place define_module calls to the very end of the define_start section.

// Comm module

DEFINE_MODULE ‘TOA_D901_COMM’ Comm1(vdvDevice, dvDevice)

// Touch panel module

DEFINE_MODULE ‘TOA_D901_UI’ TouchPanelA(vdvDevice, dvTouchPanel,

nTP_BUTTONS,nTP_TEXT,nTP_LEVELS)

Upon initialization the AMX Comm module will communicate with the D-901 and information will be exchanged.

## Command Interface

The UI module controls the audio processor via command events (NetLinx command send_command) sent to the COMM module. The commands supported by the COMM module are listed below.

* Denotes an extended command beyond the basic API.

| Command | Description |
| --- | --- |
| *BAUD=<value> | Changes the AMX baud rate. This command does not change the device’s baud rate. <value>: 9600, 19200, 38400 = supported baud rates BAUD=9600 (default at reboot) |
| *BAUD? | Query the AMX baud rate. This command does not retrieve the device’s baud rate. BAUD? |
| *BUS=<id>:<sdirection>:<schannel>:<ddirection>:<dchannel>:<value> | Sets the bus assignment of an input source to an output destination On or Off. <id>: 1 = Always 1 <sdirection>: I = Input M = Mic <schannel>: 1..12 = if <sdirection> is Input - = if <sdirection> is Mic <ddirection>: O = Output (letter O) M = Mic <dchannel>: 1..8 = if <ddirection> is Output - = if <ddirection> is Mic <value>: 0 = Off 1 = On BUS=1:I:2:O:3:1 Note: You cannot have both <sdirection> and <ddirection> as Mic at the same time. |
| *BUS?<id>:<sdirection>:<schannel>:<ddirection>:<dchannel> | Query for the state of the bus. <id>: 1 = Always 1 <sdirection>: I = Input M = Mic <schannel>: 1..12 = if <sdirection> is Input - = if <sdirection> is Mic <ddirection>: O = Output (letter O) M = Mic <dchannel>: 1..8 = if <ddirection> is Output 1 = if <ddirection> is Mic BUS?1:I:2:O:3 Note: You cannot have both <sdirection> and <ddirection> as Mic at the same time. |
| DEBUG=<value> | Enables or disables the debug feature. <value>: 0 = off 1 = on DEBUG=1 |
| DEBUG? | Request the state of the debug feature. DEBUG? |
| DEVICE_SCALE=<value> | Enables or disables the use of the device specific level values. The API supports level values from 0..100% but it might be necessary to use device specific values in certain cases. <value>: 0 = disabled (not using device specific level values / using API values) 1 = enabled (using device specific level values / not using API values) DEVICE_SCALE=0 (default at reboot) |
| DEVICE_SCALE? | Query the device scale status. DEVICE_SCALE? |
| *FILTER=<id>:<direction>:<channel>:<value> | Sets the high-pass filter On or Off. <id>: 1 = Always 1 <direction>: I = Input <channel>: 1..12 = valid input numbers <value>: 0 = Off - = On FILTER=1:I:3:1 |
| *FILTER?<id>:<direction>:<channel> | Query the current setting for the high-pass filter. <id>: 1 = Always 1 <direction>: I = Input <channel>: 1..12 = valid input numbers FILTER?1:I:3 |
| LEVEL=<id> :<direction> :<channel> :<value>[<step>] | Adjusts the level value. <id> : 1 = Always 1 <direction>: O = Output (letter O) I = Input <channel> : * = All 1..8 = Output number 1..12 = Input number <value> : 0..100 = API level value 0..63 = Device specific level units. + = increment level value - = decrement level value <step> : 0..30 = Optional additional step value. If not used, an additional step value of 0 will be used. Only used with +/-. Examples: LEVEL=1:I:1:50 (sets the Input 1 level to 50) LEVEL=1:I:*:20 (sets all 12 inputs to level 20) LEVEL=1:O:3:+ (increment Output 3 level by 1 unit) LEVEL=1:O:3:+1 (increment Output 3 level by 2 units) Note: The optional additional step value is used only with increment/decrement. If used, the additional step value will increase the rate at which the +/- adjustments will increment/decrement. If not used, an additional step value of 0 is assumed which means that the +/- will increment/decrement by 1 unit. Note: The +/- steps are always device scale steps, even when using API scale feedback. |
| LEVEL?<id> :<direction> :<channel> | Query the level value. <id> : 1 = Always 1 <direction>: O = Output (letter O) I = Input <channel> : * = All 1..8 = Output number 1..12= Input number LEVEL?1:I:1 |
| *LEVEL_MATRIX=<id>:<sdirection>:<schannel>:<ddirection>:<dchannel>:<value>[<step>] | Adjusts the cross-point matrix levels. <id>: 1 = Always 1 <sdirection>: I = Input M = Mic <schannel>: 1..12 = if <sdirection> is Input - = if <sdirection> is Mic <ddirection>: O = Output (letter O) M = Mic <dchannel>: 1..8 = if <ddirection> is Output - = if <ddirection> is Mic <value>: 0..100 = API level values 0..70 = Device specific level units. + = increment - = decrement <step>: 0..15 = Optional additional step value. If not used, an additional step value of 0 will be used. Only used with +/-. Examples: LEVEL_MATRIX=1:I:2:O:3:50 (Input 2 x Output 3 level at 50) LEVEL_MATRIX=1:M:1:O:2:+ (Mic Bus x Output 2 level up 1 unit). LEVEL_MATRIX=1:M:1:O:2:+3 (Mic Bus x Output 2 level up 4 units). Note: The optional additional step value is used only with increment/decrement. If used, the additional step value will increase the rate at which the +/- adjustments will increment/decrement. If not used, an additional step value of 0 is assumed which means that the +/- will increment/decrement by 1 unit. Note: You cannot have both <sdirection> and <ddirection> as Mic at the same time. Note: The +/- steps are always device scale steps, even when using API scale feedback |
| *LEVEL_MATRIX?<id>:<sdirection>:<schannel>:<ddirection>:<dchannel> | Query the current cross-point matrix level value. <id>: 1 = Always 1 <sdirection>: I = Input M = Mic <schannel>: 1..12 = if <sdirection> is Input - = if <sdirection> is Mic <ddirection>: O = Output (letter O) M = Mic <dchannel>: 1..8 = if <ddirection> is Output 1 = if <ddirection> is Mic LEVEL_MATRIX?1:I:2:O:3 Note: You cannot have both <sdirection> and <ddirection> as Mic at the same time. |
| *LINE=<id>:<slot>:<line>:<value> | Sets the line (stereo) select status. <id>: 1 = Always 1 <slot>: 1..6 = Input slot number <line>: 1..4 = line numbers <value>: 0 = Off - = On LINE=1:2:3:1 Note: Slot 1 means input ch1/ch2 … Slot 6 means input ch11/ch12. |
| *LINE?<id>:<slot>:<line> | Query the line status. <id>: 1 = Always 1 <slot>: 1..6 = Input slot number <line>: 1..4 = line numbers LINE?1:2:3 |
| MUTE=<id> :<direction> :<channel> :<value> | Mutes or un-mutes a specific channel. <id> : 1 = Always 1 <direction>: O = Output (letter O) I = Input <channel> : * = All 1..8 = Output number 1..12= Input number <value> : 0 = mute off 1 = mute on MUTE=1:I:1:0 |
| MUTE?<id> :<direction> :<channel> | Query the mute settings of a specific channel. <id> : 1 = Always 1 <direction>: O = Output (letter O) I = Input <channel> : * = All 1..8 = Output number 1..12= Input number MUTE?1:I:1 |
| *ONLINE? | Query the state of communications between AMX and the D-901. This online state gets updated only once, whenever valid communications exist between AMX and the D-901 and will reflect from then on Online. This online state will not be updated again unless the AMX system is rebooted. There is no constant check, once the communication module has determined a valid online state, for any further connection interruptions. ONLINE? |
| PASSTHRU=<value> | Allows user the capability of sending commands directly to whatever unit is attached without processing by the NetLinx module. User must be aware of the exact protocol implemented by the unit to use this command. This gives the user access to features that may not be directly supported by the module. The communication module does not add any characters to the passthru string. For more details see “Adding Functions to Modules” section at the end of this document. <string> : string to send to unit PASSTHRU=RESET |
| RECALL=<id>:<preset> | Recall a preset. After each preset recall, the communication module will query the device in order to get the latest/new status information. See “Programming Notes” section. If you do not want to receive this feedback information after recalling a preset, simply send a RECALL? query immediately following your initial RECALL= command. <id> : 1 = Always 1 <preset>: 1..16 = number of preset to recall RECALL=1:2 |
| *RECALL? | Query for the last preset recalled. RECALL? |
| VERSION? | Query the communication module version. VERSION? |

Table 1 – Send Command Definitions

## String Feedback

The NetLinx COMM module provides feedback to the User Interface module for the audio processor via string events. The strings supported are listed below.

| String |  |  | Description |  |  |
| --- | --- | --- | --- | --- | --- |
| BAUD=<value> | Feedback for the AMX baud rate. This command does not report the device’s baud rate. <value>: 9600, 19200, 38400 = supported baud rates BAUD=9600 (default at reboot) |  |  |  |  |
| BUS=<id>:<sdirection>:<schannel>:<ddirection>:<dchannel>:<value> | Feedback for the bus assignment of an input source to an output destination On or Off. <id>: 1 = Always 1 <sdirection>: I = Input M = Mic <schannel>: 1..12 = if <sdirection> is Input - = if <sdirection> is Mic <ddirection>: O = Output (letter O) M = Mic <dchannel>: 1..8 = if <ddirection> is Output - = if <ddirection> is Mic <value>: 0 = Off 1 = On BUS=1:I:2:O:3:1 Note: You cannot have both <sdirection> and <ddirection> as Mic at the same time. |  |  |  |  |
| DEBUG=<value> |  |  |  | Feedback on the state of the debug feature. <value>: 0 = off 1 = on DEBUG=1 |  |
| DEVICE_SCALE=<value> |  | Feedback for the device scale feature. <value>: 0 = disabled (not using device specific level values / using API values) 1 = enabled (using device specific level values / not using API values) DEVICE_SCALE=1 |  |  |  |
| ERROR - <message> | Unsolicited feedback received from the communication module whenever a problem is encountered. <message>: string describing the error message. ERROR - Invalid Mute Channel. |  |  |  |  |
| FILTER=<id>:<direction>:<channel>:<value> | Feedback for the high-pass filter On or Off. <id>: 1 = Always 1 <direction>: I = Input <channel>: 1..12 = valid input numbers <value>: 0 = Off - = On FILTER=1:I:3:1 |  |  |  |  |
| LEVEL=<id> :<direction> :<channel> :<value> | Feedback for the level value. <id> : 1 = Always 1 <direction>: O = Output (letter O) I = Input <channel> : 1..8 = Output number 1..12 = Input number <value> : 0..100 = API level value 0..63 = Device specific level units. LEVEL=1:I:1:50 |  |  |  |  |
| LEVEL_MATRIX=<id>:<sdirection>:<schannel>:<ddirection>:<dchannel>:<value> | Adjusts the cross-point matrix levels. <id>: 1 = Always 1 <sdirection>: I = Input M = Mic <schannel>: 1..12 = if <sdirection> is Input - = if <sdirection> is Mic <ddirection>: O = Output (letter O) M = Mic <dchannel>: 1..8 = if <ddirection> is Output - = if <ddirection> is Mic <value>: 0..100 = API level values 0..70 = Device specific level units. LEVEL_MATRIX=1:I:2:O:3:50 (Input 2 x Output 3 level at 50) |  |  |  |  |
| LINE=<id>:<slot>:<line>:<value> | Feedback for the line (stereo) select status. <id>: 1 = Always 1 <slot>: 1..6 = Input slot number <line>: 1..4 = line numbers <value>: 0 = Off - = On LINE=1:2:3:1 Note: Slot 1 means input ch1/ch2 … Slot 6 means input ch11/ch12. |  |  |  |  |
| MUTE=<id> :<direction> :<channel> :<value> | Feedback for the mute state. <id> : 1 = Always 1 <direction>: O = Output (letter O) I = Input <channel> : * = All 1..8 = Output number 1..12= Input number <value> : 0 = mute off 1 = mute on MUTE=1:I:1:0 |  |  |  |  |
| ONLINE<state> | Feedback for the online state. This online state gets updated only once, whenever valid communications exist between AMX and the D-901 and will reflect from then on Online. This online state will not be updated again unless the AMX system is rebooted. There is no constant check, once the communication module has determined a valid online state, for any further connection interruptions. <state>: 0 = Offline 1 = Online ONLINE=1 |  |  |  |  |
| POWER=<state> | Feedback received whenever the device’s front panel Power button is turn On. <state>: 1 = On POWER=1 |  |  |  |  |
| RECALL=<id>:<preset> | Feedback for the last recalled preset. <id> : 1 = Always 1 <preset>: 1..16 = number of preset to recall RECALL=1:2 |  |  |  |  |
| RETRIEVING | Feedback received right before the communication module will start quering for the latest status information. For more details see “Programming Notes” section. RETRIEVING |  |  |  |  |
| VERSION=<ver> | Feedback for the communication module version number. <ver> string VERSION=1.0 |  |  |  |  |

Table 2 – String Feedback Definitions

## Device Notes

- Make sure the device’s control method is set to ‘remote’ and not ‘PC control’. By default, the device is configured from factory, for ‘PC control’. To change the control method press ‘Utility’ and rotate the setting knob on the front panel of the device until RS-232C is displayed and select ‘remote’ mode.

## Programming Notes

- The communication module implements a queue for sending commands to the device. A command will wait for the previous command to receive a reply before it is sent. If a command fails to receive a reply within a timeout period the next command in the queue is sent at that time. The timeout period is 1 second.

- The communication module does not constantly poll the device. The device provides unsolicited feedback whenever a supported feature has changed. The communication module will however, query for all supported features each time a preset is recalled or the device is turned On from the front panel button. The start of this update is indicated by the RETRIEVING feedback sent by the communication module to the user-interface. This update takes about 15 seconds and it is recommended that no commands be sent during that time. Any command sent during this time will be processed whenever the update has completed.

- The sample user-interface and touch panel file are provided as an example. There is no expectation that those files will be used as is in an actual installation.

- The sample user-interface and touch panel file do not implement all the features supported in the communication module. If additional features need to be supported in your installation, then additional user-interface code must be developed and changes to the touch panel must be made according to your needs.

- In order to map the device level units to dB see the included D901 232CProtocol v2_00E.pdf file

(page 12). Device specific level values must be used (see DEVICE_SCALE= command) in order for the communication module to report the device level units, which in turn can be mapped to dB values according to the table found in the .pdf file. This is applicable only to the LEVEL= and LEVEL_MATRIX= commands and queries. Upon reviewing the tables found in the .pdf file, the table titled “Position vs. Gain Table for Fader” applies to the LEVEL= command, and the table titled “Value vs. Gain Table for Crosspoint gain” applies to the LEVEL_MATRIX= command. The decimal position in those tables is the device level unit reported by the communication module. So, for example, a position value of 4 means –42 dB in the first table.

## Adding Functions to Modules

### Commands to the device

This module supplies a mechanism to allow additional device features to be added to software using the module.   This is the PASSTHRU command, which allows protocol strings to be passed through the module. The device-specific protocol must be known in order to use this feature.

As an example, suppose that a module for a projector has not implemented the 'white balance adjustment' feature.   The command that the projector protocol requires is 03H, 10H, 05H, 14H, followed by a checksum.   The documentation for the PASSTHRU command specifies that the module will automatically generate the checksum.   In this case, the following string should be sent from

the UI code to implement 'white balance adjustment'.

send_command vdvDevice,"'PASSTHRU=',$03,$10,$05,$14"

The reason to use PASSTHRU instead of sending a protocol string directly to the device port is that the device may require command queuing, calculation of checksums, or other internal processing, which would not be done if the string was sent directly. Because of this, it is best to filter all communication TO the device through the module. (The module documentation will indicate any processing that will be automatically done to the PASSTHRU string like checksum calculation.)

### Additional Feedback from the device

The module documentation indicates what feedback is provided.  If additional feedback is required, a CREATE_BUFFER for the device must be implemented in the user code to process the strings from the device manually. Note that the module will still be processing the response strings independently

and sending the interpreted feedback up to the user code.